EXPERIMENT 1B

Name:- S Suresh

Reg. No. :- 192412178

Program Code:

clc;

clear;

close all;

%% QUESTION B: TE10 VERSUS TM11 MODE

% WR-90 Rectangular Waveguide

a = 22.86e-3;       % Width (m)

b = 10.16e-3;       % Height (m)

L = 100e-3;         % Length (m)

f = 10e9;           % Operating frequency (Hz)

c = 3e8;            % Speed of light (m/s)

mu0 = 4*pi*1e-7;

eps0 = 8.854e-12;

omega = 2*pi*f;

k0 = 2*pi*f/c;

%% 1. CUTOFF FREQUENCIES

% TE10 mode

fc_TE10 = c/(2*a);

% TM11 mode

fc_TM11 = (c/2)*sqrt((1/a)^2 + (1/b)^2);

fprintf('\n--- CUTOFF FREQUENCIES ---\n');

fprintf('TE10 cutoff frequency = %.4f GHz\n',fc_TE10/1e9);

fprintf('TM11 cutoff frequency = %.4f GHz\n',fc_TM11/1e9);

%% 2. PROPAGATION CONSTANTS

% TE10 propagation constant

if f > fc_TE10

beta_TE10 = k0*sqrt(1-(fc_TE10/f)^2);

else

beta_TE10 = 0;

end

% TM11 propagation constant

if f > fc_TM11

beta_TM11 = k0*sqrt(1-(fc_TM11/f)^2);

else

beta_TM11 = 0;

end

fprintf('\n--- PROPAGATION CONSTANTS ---\n');

fprintf('TE10 beta = %.4f rad/m\n',beta_TE10);

fprintf('TM11 beta = %.4f rad/m (real propagation)\n',beta_TM11);

%% 3. WAVEGUIDE COORDINATES

Nx = 100;

Ny = 80;

Nz = 100;

x = linspace(0,a,Nx);

y = linspace(0,b,Ny);

z = linspace(0,L,Nz);

[X,Y] = meshgrid(x,y);

%% 4. TE10 ELECTRIC FIELD DISTRIBUTION

Ey_TE10 = sin(pi*X/a);

figure(1);

surf(X*1000,Y*1000,Ey_TE10,'EdgeColor','none');

colorbar;

xlabel('Width x (mm)');

ylabel('Height y (mm)');

zlabel('Normalized Ey');

title('TE10 Electric Field Distribution');

view(45,30);

grid on;

%% 5. TE10 MAGNETIC FIELD DISTRIBUTION

Hx_TE10 = -beta_TE10/(omega*mu0)*sin(pi*X/a);

Hz_TE10 = -pi/(omega*mu0*a)*cos(pi*X/a);

H_TE10 = sqrt(Hx_TE10.^2 + Hz_TE10.^2);

figure(2);

surf(X*1000,Y*1000,H_TE10,'EdgeColor','none');

colorbar;

xlabel('Width x (mm)');

ylabel('Height y (mm)');

zlabel('|H| (A/m)');

title('TE10 Magnetic Field Distribution');

view(45,30);

grid on;

%% 6. TM11 ELECTRIC FIELD DISTRIBUTION

Ez_TM11 = sin(pi*X/a).*sin(pi*Y/b);

figure(3);

surf(X*1000,Y*1000,Ez_TM11,'EdgeColor','none');

colorbar;

xlabel('Width x (mm)');

ylabel('Height y (mm)');

zlabel('Normalized Ez');

title('TM11 Longitudinal Electric Field Distribution');

view(45,30);

grid on;

%% 7. TM11 MAGNETIC FIELD DISTRIBUTION

% Normalized spatial field pattern for TM11

% Valid as a cross-sectional pattern below cutoff;

% it is not a propagating-wave solution at 10 GHz.

Hx_TM11 = (pi/(omega*mu0*b))*cos(pi*X/a).*sin(pi*Y/b);

Hy_TM11 = -(pi/(omega*mu0*a))*sin(pi*X/a).*cos(pi*Y/b);

H_TM11 = sqrt(Hx_TM11.^2 + Hy_TM11.^2);

figure(4);

surf(X*1000,Y*1000,H_TM11,'EdgeColor','none');

colorbar;

xlabel('Width x (mm)');

ylabel('Height y (mm)');

zlabel('|H| (A/m)');

title('TM11 Magnetic Field Pattern');

view(45,30);

grid on;

%% 8. COMPARISON OF CUTOFF FREQUENCIES

figure(5);

bar([fc_TE10 fc_TM11]/1e9);

set(gca,'XTickLabel',{'TE10','TM11'});

ylabel('Cutoff Frequency (GHz)');

title('TE10 versus TM11 Cutoff Frequency');

grid on;

%% 9. MODE PROPAGATION DECISION

fprintf('\n--- MODE COMPARISON ---\n');

if f > fc_TE10

fprintf('TE10: PROPAGATES at 10 GHz.\n');

else

fprintf('TE10: Below cutoff.\n');

end

if f > fc_TM11

fprintf('TM11: PROPAGATES at 10 GHz.\n');

else

fprintf('TM11: DOES NOT PROPAGATE at 10 GHz.\n');

end

fprintf('\nConclusion:\n');

fprintf('TE10 cutoff is below 10 GHz.\n');

fprintf('TM11 cutoff is above 10 GHz.\n');

OUTPUT